### SECTION 6

## EQUIPMENT

### 1.	PARACHUTES - GENERAL

1.1.	All sport parachutists intending to make a descent are to be equipped with a minimum of two airworthy parachutes attached to a common harness, except Student Tandem Parachutists who must be attached to a Tandem Instructor.  Jumpmasters or instructors not intending to make a parachute descent must be equipped with a minimum of one airworthy parachute.

1.2.	Parachutes may only be used if they are manufactured for Sport Parachutists or Military Parachutists, by recognised parachute equipment manufacturers or riggers with the necessary qualifications.

1.3.	Any modifications to parachute equipment must be carried out by a parachute equipment manufacturer or a rigger with the necessary qualifications. Modifications to equipment, including changes to manufacturer’s specifications, used by Student Parachutists, including Tandem equipment, must be approved by the BPA, via Riggers’ Committee, before use.

N.B.	BPA Form 209 complete with relevant drawings should be submitted for modifications to equipment used by Student Parachutists.

1.4.	Changes to manufacturer’s specifications on Tandem equipment must be approved by the BPA (via Riggers’ Committee and STC).

### 2.	EQUIPMENT USED BY STUDENT PARACHUTISTS

All equipment, other than main parachutes used by Student Parachutists must be acceptable to the BPA (via Riggers’ Committee and STC).

2.1.	AFF & Category System

2.1.1.	Main parachutes must be ram-air type and must be large and docile in relation to the parachutist’s weight and height. The main and reserve parachutes must be contained in  ‘piggyback’ type containers.

2.1.2.	Where static line deployment is used, the static line must be continuous from deployment bag to the point of attachment in the aircraft.

2.1.3.	The following Wing Loading criteria should be applied to main canopies:

a.	First Jump ‘Solo’ Student Parachutists - not to exceed 0.8 lbs/sq.ft.

b.	‘Solo’ Student Parachutists having completed at least one ‘solo’ descent - not to exceed 0.85 lbs/sq.ft.

N.B.	Exceptions to the above are permitted where a Canopy’s Manufacturer has published advice that a higher wing loading is suitable. The manufacturer’s higher limit may then apply.

2.1.4.	Equipment must be fitted with a reserve static line (RSL) and an automatic activation device (AAD), which must be switched on prior to any descent.

SEC 6/PAGE 1/AUG 2008

    - Category System freefall equipment must be ripcord deployed, up to at least the completion of Category 6.

2.1.6.	Accelerated freefall equipment may be either ripcord or throwaway deployed, and must be able to be activated from either side, unless the Student Parachutist is making a Level 8 or Consolidation descent.

2.2.	Tandem

    - Tandem equipment is not classified as Student Parachutist equipment, however it is equipment used by Student Parachutists. Therefore only Tandem equipment acceptable to the BPA (via Riggers’ Committee and STC) may be used.

    - Tandem equipment must be fitted with an AAD specifically designed for tandem equipment. If the equipment is not specifically designed to accept an AAD, the initial installation must be carried out by an Advanced Rigger. In the case of a Cypres AAD the Advanced Rigger must be approved by Airtec.

    - Tandem equipment (excluding the Student Harness – see 2.2.5. below) must be packed and fully inspected every 6 months (BPA Forms 112A & 112D). Also, the Tandem equipment, excluding the reserve canopy, must be inspected every 100 descents (BPA Form 112B & 112C).

N.B(1).	The 100 descent inspection may take place between 100 and 110 descents.

N.B(2).	The 100 jump inspection requirements can be found on BPA Form 112C.

N.B(3).	BPA Forms 112A, 112B, 112C, 112D & 112E must be retained with the Tandem equipment documentation.

    - The 100 descent inspection must by carried out by Advanced Packer (Tandem), or a rigger.

    - The Student Tandem harness must be inspected monthly and certified as serviceable, by a Tandem Instructor, Advanced Packer (Tandem), or rigger (BPA Form 112D).

    - The owner of the Tandem equipment must maintain a Log Card/record of all descents, which must be available for the 100 jump and 6 monthly inspections/repacks (BPA Form 112B).

    - The owner of the Tandem equipment must maintain a log of repairs, modifications and component replacements, which must be available for the 100 jump and 6 monthly inspections/repacks (BPA Form 112E).

2.2.8.	Parachutists who are not Tandem Instructors (TI), or TI candidates undergoing evaluation, may not jump Tandem equipment.

3.	PERSONAL CLOTHING AND EQUIPMENT

All parachutists will be equipped with suitable clothing and equipment. ‘B’ Licence parachutists are responsible for the safe condition of personally owned equipment that they may use, and for ensuring that such equipment is serviced, checked and inspected as appropriate.

SEC 6/PAGE 2/MAR 2012

3.1.	Helmets

    - Student and BPA ‘A’ Licence parachutists must wear a suitably sturdy open faced, hard protective helmet without a peak.

N.B.	The ‘Raleigh Burner’ helmet is not acceptable for use by Student Parachutists.

3.1.2.	Student Tandem Parachutists must wear a ‘French Type’ ribbed helmet.

3.1.3.	BPA ‘B’ Licence holders may wear hard protective helmets acceptable to the CCI/Team Leader.

    - BPA ‘C’ Licence holders may wear headgear acceptable to the CCI/Team Leader.

    - Helmets are to be worn throughout the entire parachute descent.

3.2.	Footwear which is suitable for the planned descent.

3.3.	Gloves (if worn) must allow easy operation of emergency equipment.

3.4.	All BPA ‘A’ Licence parachutists and above are required to carry a serviceable knife about their person, or on their equipment.  The knife should be readily accessible.

4.	INSTRUMENTS

4.1.	Altimeters must be worn by all Student Parachutists (other than Tandem Student Parachutists). Also, altimeters must be worn by BPA Licenced parachutists carrying out planned delayed openings of 15 seconds or more.

4.2.	The minimum instrument is a serviceable non-sensitive visual read out altimeter, securely mounted in a suitable position.

4.3.	Instruments should be positioned so as not to interfere with the use of other parts of the equipment.

4.4.	Tandem and AFF instructors must be equipped with an audible altimeter when carrying out Tandem and/or AFF instructional descents.

  - AFF and Category System Student Parachutists must be equipped with a radio receiver for at least the first three descents.

### 5.	AUTOMATIC ACTIVATION DEVICES

BPA ‘A’ Licence Parachutists must use equipment fitted with an operational AAD, which must be switched on prior to any descent.

N.B.	Also see sub-paras 2.1.5. and 2.1.4. above and Section 13 (Displays) para 4.3.

6.	CAMERAS

6.1.	Cameras may only be used by BPA ‘C’ Licence parachutists after inspection and approval of a CCI.

SEC 6/PAGE 3/MAR 2012

  - Cameras must be securely fitted.   Only in cases of emergency should they be jettisoned.

- AUXILIARY EQUIPMENT

Equipment such as; GPS, flags, smoke brackets etc, may be actively used by BPA ‘C’ Licence parachutists with permission of the CCI/Team Leader. They must be securely fitted to the parachutist. Only in the case of emergency should they be jettisoned.

N.B.	Skysurfing boards may only be used by SS1 Grade parachutists or above, or parachutists training for SS1 (see Section 2, para 6.8).

### 8.	PARACHUTE PACKING

- Student Equipment.

All Student main parachutes are to be packed under the supervision of a person holding a BPA Packing Certificate for the parachute being packed.

  - Main Parachutes

BPA ‘B’ Licence parachutists and above and holders of JM1 Grade qualification are responsible for the packing of the main parachutes they are jumping.

8.3.	Reserve Parachutes

8.3.1.	Reserve parachutes may only be inspected and packed by riggers or Advanced Packers (AP).

N.B.(1)	The Training Syllabus for Advanced Packer Courses can be found on BPA form 169.

N.B.(2)	Advanced Packers are subject to an annual currency certification at the time of membership renewal. This certification will be displayed on the BPA membership card.

8.3.2.	Reserve Parachute Packers must keep a log of all reserves packed.

8.4.	Where manufacturers’ requirements conflict with those of the BPA, the BPA requirements will apply.

8.5.	Reserve parachutes that have been packed in a foreign country, in a manner acceptable to the parachuting organisation of that country, may be jumped at a BPA Club for up to 6 months from the date of that packing. This is provided that the parachuting organisation of that foreign country allows 6 months validity for a reserve repack; otherwise the foreign country’s lesser time will apply. In the case of visiting foreign parachutists the length of time may be up to 12 months, depending on the repack cycles permitted in their country.

8.6.	Prior to packing reserve parachutes (excluding Tandem reserve parachutes – see para 8.7. below) they must be inspected and certified as serviceable on the appropriate Record of Inspection/Inspection Check List. (BPA Forms 112, or 113).

SEC 6/PAGE 4/MAR 2012

8.7.	Prior to packing Tandem reserve parachutes, the Tandem system, including main parachute, reserve parachute, container, instructor harness, AAD and drogue, must be inspected and certified as serviceable on the Tandem Record of Inspection/Check List (BPA Form 112A). This form must be retained with the Tandem system documentation.

8.8.	Packing/record cards must also be maintained for all reserve parachutes.

8.9.	The pull force of a reserve ripcord on piggyback equipment may be test pulled by the user after the reserve has been packed.  The user should then sign the Inspection Check List to indicate that he/she has made the test pull. In all cases, the packer must test pull the reserve ripcord with a scale and sign for the recorded poundage on the Inspection Check List.  The maximum acceptable pull force is 22 lbs.

8.10.	Repack Cycles.

Parachutes must be packed at least every 6 calendar months.

  - Reserve Sealing Thread

Only Riggers, Advanced Packers and packers with the appropriately endorsed packing certificate may seal reserves and must use a method acceptable to the BPA Riggers Sub-Committee. The rigger/packer should use a method of identification on their seal that is identifiable to him/her. Seal thread must be a thread manufactured to break at no more than 6lbs. The sealing of reserves is optional.

N.B.	The acceptable BPA reserve sealing method can be found on BPA form 215.

### 9.	PACKING CERTIFICATES

Approved Packing Certificates (Main parachutes). Holders of these certificates are certified to assemble and pack specified parachutes and deployment devices into specific containers. These may be issued or endorsed by Parachute Riggers/Instructors (excluding CSBIs, TBIs or AFFBIs).

10.	PACKING RECORDS

10.1.	Clubs will maintain a record for each parachute in use as club equipment.

10.2.	Student Parachutists will maintain a record for their personal parachutes.

10.3.	Packing records will log every packing, re-packing and modification to the parachutes to which they relate.

10.4.	Records must incorporate the date of packing and signature of the holder of a relevant packing certificate, at each of the stages of packing that requires inspection.

SEC 6/PAGE 5/JUNE 2012