The 10 Point Kit on Check.

How to look after yourself after AFF.

This check is advised when you put your gear on ready to jump, make sure you are ready well before emplaning so you have time to do this check yourself and then also get checked by an experienced jumper on the flight line. Don’t be the one running after the aircraft with your leg straps undone. Remember “We wait for the aircraft, the aircraft does not wait for us”

1). Leg Straps done up correctly and the ends tucked away.

2). Cut Away Pad in place and accessible.

3). Reserve Handle in place and accessible.

4). Chest Strap routed correctly and the end tucked away.

5). Both Three Ring Releases correctly assembled and the yellow teflon cable going

through the nylon loop. Loop in good condition.

6). RSL attached and done up tight.

7). Altimeter secured correctly and set to zero.

8). Main Handle in place and accessible.

9). Goggles you have them and their tight enough.

10). Helmet you have it and its tight enough with the excess strap secured with an

elastic band.