Skydive Spain

Throwaway Brief

Aids:  Throwaway Rig, Pull Up

Compulsory Headings

Subjects

1). Documentation

2). Introduction

3). How it Works

4). Packing Instructions

5). Pull Procedure

6). Emergencies &

Malfunctions

7). Compatible Equipment

Check log book, should have graduated AFF or finished Cat 8 qualifier jump on RAPS.

The aim is to teach how to pack and operate a throwaway deployment system. The use is allot easier to pack and more efficient than RC.

Include: BOC Pocket

Pilotchute differences

Bridal line with pin

Show packing of Pin, Bridal line & Pilotchute as per Skydive Spain’s packing SOPs.

Show how to grip toggle when pulling. How to extract and throw for maximum efficiency

Keep shoulders level

Should have a more instant deployment

Lazy pulls:	Check that it is out of BOC

Good look over shoulder

Bridal line around Arm / Leg / Neck,

try two times to remove before going through reserve drills.

If right arm is incapacitated, reserve drills can be operated by left arm.

Freefly friendly or not.